Experiment X

Cosmic Rays

You will be investigating given datasets from muon detectors that were taken under various conditions, then taking your own data under two of these conditions and comparing it to the given datasets. There are setups that have been created to recreate the conditions that the dataset was taken in.

X.1 History and theory

Cosmic rays, which are relativistic charged particles that primarily originate from stars and supernovae, are constantly entering Earth’s atmosphere. These particles, primarily protons and alpha particles, typically carry energy around 1 GeV, although they can potentially carry up to 1011 GeV. When these particles enter the atmosphere, they collide with nuclei in the upper atmosphere. These collisions release charged and neutral particles called mesons, primarily pions and kaons. Charged pions and kaons will decay after 1E-8 seconds and are both able to decay  into muons, with pions decaying into them approximately 99.98% of the time and kaons decaying into them approximately 63.5% of the time. Due to this, 80-90% of muons seen in Earth’s atmosphere come from pion decay. A muon will decay after 2.2E-6 seconds, but muons with energy greater than 2.4 GeV will be relativistic enough to travel 15 km before they decay. These are the muons that are detected in the CosmicWatch detector, as they are able to travel far enough to reach sea level.

X.2 Pre-Laboratory Preparation

Please complete this prior to your assigned lab section.

Download the python program located in canvas, then read the parameters and descriptions located at the top of each program. Download the given datasets also located in canvas.

Investigate shielding materials for cosmic rays and muons. Are the best shielding materials for muons and other cosmic ray particles the same? If not, what property makes a material suitable for specifically blocking muons?

X.3 Experimental Procedure

X.3.1 Important Considerations

Please be aware of these important aspects while doing the experiment.

- You will need to find a time and way to go back to the lab and collect the data taken by the muon detectors.

- Be careful when inserting the detectors into the housings. Wash your hands immediately after handling the lead bricks.

- Ensure that there are no files on the SD card before starting data collection, and delete any files on the SD card after data is collected.

- Whenever the SD card is being inserted into/taken out of the detector, hold down the grey reset button located on the front on the detector.

- Always eject the SD card from your computer before removing it.

X.3.2 Experimental Setup

WIP

X.3.3 Experiment 1: Analysis of datasets

- Download the six datasets and the python code from canvas. There are three datasets for each detector, and each detector had data recorded under the same three conditions. Plot each of the datasets in order to get familiar with the code, and note any trends that you see in the histograms and the count rates. The python code is clearly documented if there is any confusion on where the values in the histogram or the rate plot come from.

- Use the information that was gathered in the prelab to predict the shielding conditions for each dataset. The three options for potential shielding conditions are listed at the top of the python code. Be sure to document the process of determining the shielding conditions.

X.3.4 Experiment 2: Measuring two shieldings

- Choose at least two of the shielding conditions to take data with. Put each detector in one of the housings, and carefully stack the lead brick(s) on the housing. After the detectors are set up, press the grey reset button to start a new dataset that is taken entirely under the housings.

- Allow the detectors to run for at least twelve hours to ensure that trends in the data are visible. After the detectors have run for at least twelve hours, take out the microSD card while holding the grey reset button. There will be numbered text files on the microSD card, and the file with the largest size will be your data.

- Open the program and follow the instructions to plot both of your datasets. Take note of the count rates located in the botton left of the rate plots as they are very useful for analysis.

X.4 Results, analysis, and deliverables

You will turn in your predictions for the shielding conditions of each of the provided datasets, as well as the process for arriving at these predictions. After plotting your experimental data, compare it to the given datasets and discuss whether or not the given datasets agree with your experimental ones. You must turn in both of the histograms and both of the rate plots for your experimental datasets. Be sure to discuss any potential causes for inconsistencies between the given datasets and the datasets that were taken during the experiment.

Questions

There are two sets of plots located in canvas, name and name. The first of these datasets is data taken by the detector while on a plane. The count rate clearly increases by a massive amount as the plane ascends, but can you think of any other factors that could cause the count rate to erroneously increase under these conditions?

The second of these are rate plots and histograms that were taken with the detectors under three lead bricks. Compare one of these to the dataset from the same detector that you identified as being taken under two lead bricks in part one. Do you see anything unexpected? What could explain any unexpected similarities or differences that you see between the two datasets?